EBARA · GLOBAL SOURCING OFFICE

# Supplier Visit Report

Hebei Huatong Wires & Cables Group Co., Ltd.

| Document Number | GSO-2608001x00 |
| --- | --- |
| Supplier | HEBEI HUATONG |
| Employee | Reinaldo Alves |
| Period | August 2026 |
| Visit Date | Aug 12, 2026 |
| Location | Tangshan, Hebei |
| Members | Reinaldo Alves, Corrado Braconi, Xu Jianping |

1.  Purpose

Due to the current shortage of DKE cable supply, EPAC is evaluating Hebei Huatong Cable Group as a potential alternative supplier of RHW-2 electrical cables. The visit aims to assess the factory’s production facilities, manufacturing capacity, quality control systems, testing laboratories, and compliance with applicable UL, CSA and ISO certifications.

The goal is to confirm Hebei Huatong’s capability to consistently meet EBARA’s technical and quality requirements. Subject to a satisfactory factory assessment, the qualification process will proceed with sample validation, followed by production orders upon successful approval.

2.  Company Information

| Legal Name | Hebei Huatong Wires & Cables Group Co., Ltd. |
| --- | --- |
| President | — |
| Established | 1993 |
| Capital | — |
| Employees | 6,000 |
| Factory Size | 200,000 m² |
| Production Capacity | — |
| Certifications | ISO9001, ISO14001, ISO45001, UL, CSA |
| Address | Luannan County, Tangshan, Hebei, China, Tangshan, Hebei, China |
| Telephone | — |
| Website | htcablewire.com |
| EBARA Track Record | RHW-2 qualification in progress |

3.  Company Overview

Hebei Huatong is a publicly listed international cable group serving oil and gas, marine, port-crane, mining and industrial applications. The presentation describes a multi-country manufacturing footprint and overseas warehouse reserves.

The supplier presents constructions aligned with European, North American and Japanese markets, potentially supporting regional product strategies.

4.  Main Products

The corporate presentation lists cables for oil and gas, marine, port cranes, mining, fixed installations, industrial and tunneling applications. The following pump-related constructions were specifically presented; all remain subject to datasheet, construction and certification validation.

| Type | Standard | Voltage | Description |
| --- | --- | --- | --- |
| H07RN8-F | EN 50525-2-21 | 450/750 V | Water-resistant European flexible cable |
| H07RN-F | EN 50525-2-21 | 450/750 V | European rubber flexible cable |
| H05BN4-F | EN 50525-2-21 | 300/500 V | European flexible cable |
| SOOW | UL 62 | 600 V | North American portable power cable |
| RHW-2 | UL 44 | 600 / 2,000 V | North American thermoset-insulated wire |
| Type W | UL 1650 | 2,000 V | Heavy-duty portable power cable |
| 2PNCT | JIS C 3306 | 600 V | Japanese pump / portable cable |

Product show room — certified cable range

Show room — export market constructions display

Insulated cores at the laying-up station

Copper conductor bobbins staged at line entry

5.  Target Products

Datasheet TDS_RHW-2_Submersible_Pump shared with the supplier before the visit. Jacket thickness deviation requires EPAC engineering confirmation before sample order.

| Product | Model | Application | Market | Requirements |
| --- | --- | --- | --- | --- |
| RHW-2 submersible pump cable | 14×4C+14×4C / 10×4C+14×4C | Submersible pump power supply | North America (US, CA) | UL 44 / CSA C22.2, 600–2000 V, 90 °C wet rating, OD per EPAC drawing |
| 2PNCT portable pump cable | 3C×3.5 mm² / 4C×5.5 mm² | Portable dewatering pumps | Japan | JIS C 3306, 600 V, rubber sheath |

6.  Visit Relevant Information

- Quality — Five separate inspection stages are applied along the cable manufacturing process, from raw material to finished product.

- Production Capacity — The rubber cable workshop reported 14,237,954 km of output and RMB 1.2 billion of value in 2025.

- Lead Time — Target production lead time is approximately 6 weeks.

- Supply Chain — EBARA orders would be manufactured at the South Korea plant; the Panama plant is dedicated to oil and gas customers (Baker Hughes, Shell).

Questions and answers

- Preventive maintenance is split into technical and routine; daily activities, including lubrication, are performed and recorded.

- Raw materials carry batch and origin identification; inspection and documentation maintain full traceability.

- Finished-product records are linked to raw-material batches; the Production Department demonstrated the complete trace-back route.

- Top three quality issues of the last 12 months — pending; the factory will provide the information in English.

- Outer-diameter stability for the 7+1 core design is monitored continuously along the full cable length; OD control limits and inspection methods to be confirmed.

- Reported supply history with Furukawa, WW and RIKEN; references for active projects to be confirmed.

7.  Certificates

Certificate copies were collected on site. Surveillance status was not independently verified during the visit; ISO 45001 evidence is the current OH&S standard.

8.  Partners

Supplies cables to Lindsay (center-pivot irrigation systems), a relevant reference for pump and water-application segments. The group also reported supply to WW Cables and to data-centre customers including NVIDIA, indicating acceptance by high-requirement global accounts.

Finished goods warehouse — export drums

Jacket surface and print marking detail

9.  Conclusion

—

10.  Appendix

| Factory entrance and administration building | Rubber cable production line — general workshop view |
| --- | --- |
| Copper conductor bobbins staged at line entry | Cable extrusion line with overhead cooling trough |
| Insulated cores at the laying-up station | Process quality control board at the workshop entrance |

| Product show room — certified cable range | Show room — export market constructions display |
| --- | --- |
| Raw material warehouse — armouring wire stock | High / low temperature conditioning chamber |
| Cold bend test rig (large cross-section) | Heat aging oven used for insulation ageing tests |

| Vertical flame test chamber (UL single-cable method) | CNAS-accredited laboratory — electrical test benches |
| --- | --- |
| Vulcanising line — continuous curing section | Tensile strength and elongation test machine |
| First sample cut — conductor and jacket inspection | Finished drum staged for pre-shipment inspection |

| Production report and traceability record sheet | Routine inspection record at the machining station |
| --- | --- |
| Finished goods warehouse — export drums | Jacket surface and print marking detail |